- Purpose: This checklist is a job aid for evaluating attainment of minimum vehicle radio equipment requirements for ACS-RACES operators to be deemed QUALIFIED FOR LOCAL DEPLOYMENT within Arlington County only. This checklist shall not be used for Mutual Aid deployments. Your personal “Go Kit” will be inspected separately using the Personal Equipment Checklist For 12-Hour Operational Period – Urban Environment, for each incident or exercise.  A passing score of 20 points is expected.  Requirements in BOLD may not be waived.

- Operator Name _______________________________Call____________ Date___________

- Evaluator Name_______________________________Call____________ Title___________

- Vehicle Make__________________ Model _________________ TAG__________________

- □ 2m FM Portable required,  ≥ 2w, 1 pt./w, manufacturer’s’ rating - max. + 5 pts.	____

- Use of an HT as your primary vehicle radio requires 12V DC power connection + an external unity gain antenna having a  clear RF path outside the vehicle.

- □ 2m radio must be frequency agile, programmable from control panel + 5 pts.	____

- Operator shall demonstrate the ability to program a new CTCSS tone and frequency.

- □ Ten memory channels on 2 meters are required as a minimum	+ 3 pts.	____

- Operator shall demonstrate that the radio is programmed to the RACES Communication Plan as a minimum.

- □ Continuous Tone Coded Squelch Encode is required	(see demo above) + 3 pts.	____

- Operator shall demonstrate the ability to access the W4AVA 146.625- repeater, with PL tone 107.2

- □ 2m FM Mobile  < 5w low, ≥ 25w high, with CTCSS, 10 memories	+ 3 pts.	____

- Use of brick amp or docking booster + HT meets desired 25w ability when powered from a fused 12V DC source.

- □ Hard-wired, fused connections to vehicle battery 1 pt. each radio up to max.+4 pts.	____

- □ Permanent antenna mounts on vehicle, 1 pt. each, up to maximum +4 points  		____

- □ Mobile 220 or 440 FM @ 10w minimum, if capable, 1 pt. per band, up to max.+2 pts.	____

- □ Portable(s) other than the primary vehicle radio, 2m, 220 or 440 FM, IF  ≥ 2 watts, ten memories / band required, + CTCSS encode and two of following power sources: 1) OEM rechargeable pack, 2) AA battery case or 3) ext. 12V DC, 1 pt. / band up to maximum +3 pts.	____

- □ Operator possesses valid GMRS License and is equipped with a Part 95 accepted portable capable of use on the 7 interstitial simplex frequencies common with FRS		+ 1 pt.	____

- □ Part 95 Type accepted GMRS portable radio is also capable of repeater use	+ 1 pt.	____

- Total Points Attained by ACS-RACES Member for this Vehicle (Pass = 20)		____

- Operator vehicle is	 □ QUALIFIED □ NOT QUALIFIED   Evaluator Initial ______

- Radio Officer Review: □Approved □Appealed  □Denied Because __________________

- ________________________________________________________________

- RO Signature: 							Call